- Beam line and saw cutting and drilling of Beams

Min specifications:

Flange size:   Max 500mm

Flange thickness:max50 mm

Web size:Max 500 mm

Web thickness:Max25mm

Beam length:12m

Drilling Size:Max Dia 50 mm

Including Infeed and outfeed conveyors 12 m each

Including cross conveyor in and out 3 OR 4 m

With marking unit .

Minimum 3 Spindles.

Measurement System.

Milling ,Tapping and countersinking .

Nesting software .

- Plate processor – Cutting and drilling of plates

Plate size = 6000 X 2000

Maximum Thickness 75 mm

One plasma Hypertherm head and one oxyacetylene head.

Max Hole diameter 40 mm

With Marking OR scribing unit

Infeed conveyor 6m OR Fixed for Gantry type.

Nesting software

- Plate rolling machine

4 roller type

Max. Plate width 2000 mm

Maximum thickness 18 mm

With infeed and out feed support

With top support

With NC System

- Profile (Section) rolling machine.

3 Roller type mounted horizontally .

Angle Rolling max 150X150 X15 mm

UPN 200 ,IPE 240 mm, H Beam 240

Pipe bending maximum 2-inch dia.

With NC system.

Request for Information:

We would appreciate receiving the following information in your OFFER.

- Detailed technical specifications for each machine.

- Lead time & Incoterms for delivery. (we prefer CIF-Dammam seaport price)

- Pricing: Please include itemized pricing for each machine, including accessories, tools, etc.  taxes, duties, and other applicable charges.

- Warranty and after-sales service terms. (whether stationed in KSA, if not then what will be the time and strategy to attend to after-sales breakdown servies)

- Availability of spare parts and service in our region. (preferably KSA or within Middle east)

- Payment terms (we prefer 180 days L/C)

- Training and support: Any provisions for operator training, maintenance support, and software upgrades.

Additionally, we would be grateful if you could share any relevant product brochures, technical data sheets, and past sales in KSA or other GCC countries demonstrating the successful deployment of these machines in similar applications.